A. Multifunktionale Pins des Élan™SC400

| Signal You Want |  | Signals You Give Up | How to Configure the Signal You Want on the Pin |  |
| --- | --- | --- | --- | --- |
| System Interface |  |  |  |  |
| BALE |  | KBD_ROW10 | Set bit 2 in the Pin Mux Register B to 1. |  |
| DBUFOE |  | GPIO_CS4 | Hardwire strap the CFG3 pin High. |  |
| DBUFRDH |  | GPIO_CS3 | Hardwire strap the CFG3 pin High. |  |
| DBUFRDL |  | GPIO_CS2 | Hardwire strap the CFG3 pin High. |  |
| MCS16 |  | KBD_ROW12 | Set bit 2 in the Pin Mux Register B to 1. |  |
| PDACK1 |  | KBD_ROW7 | Set bit 2 in the Pin Mux Register B to 1. |  |
| PDRQ1 |  | KBD_ROW8 | Set bit 2 in the Pin Mux Register B to 1. |  |
| PIRQ7 |  | KBD_COL6 | Set bit 2 of the Pin Mux Register C to 1. |  |
| PIRQ6 |  | KBD_COL5 | Set bit 2 of the Pin Mux Register C to 1. |  |
| PIRQ5 |  | KBD_COL4 | Set bit 2 of the Pin Mux Register C to 1. |  |
| PIRQ4 |  | KBD_COL3 | Set bit 1 of the Pin Mux Register C to 1. |  |
| PIRQ3 |  | KBD_COL2 | Set bit 1 of the Pin Mux Register C to 1. |  |
| PIRQ2 |  | KBD_ROW9 | Set bit 2 of the Pin Mux Register B to 1. |  |
| PIRQ1 |  | GPIO_CS7 | Set bit 2 of the Pin Mux Register A to 1. |  |
| PIRQ0 |  | GPIO_CS8 | Set bit 1 of the Pin Mux Register A to 1. |  |
| R32BFOE |  | KBD_ROW13 | Hardwire strap both the CFG1 and CFG0 pins High. |  |
| SBHE |  | KBD_ROW11 | Set bit 2 in the Pin Mux Register B to 1. |  |
| Memory Interface |  |  |  |  |
| CASH3 |  | KBD_ROW3 | Set bit 3 of the DRAM Bank x Config. Register to 1. |  |
| CASH2 |  | KBD_ROW2 | Set bit 3 of the DRAM Bank x Config. Register to 1. |  |
| CASL3 |  | KBD_ROW1 | Set bit 3 of the DRAM Bank x Config. Register to 1. |  |
| CASL2 |  | KBD_ROW0 | Set bit 3 of the DRAM Bank x Config. Register to 1. |  |
| MA12 |  | KBD_ROW6 | Set bit 3 of the DRAM Bank x Config. Register to 1. |  |
| RAS3 |  | KBD_ROW5 | Set bit 3 of the DRAM Bank x Config. Register to 1. |  |
| RAS2 |  | KBD_ROW4 | Set bit 3 of the DRAM Bank x Config. Register to 1. |  |
| Parallel Port |  |  |  |  |
| ACK | GPIO25, BVD1_B |  |  | Set field 1–0 of the Pin Mux Register B to 10. |
| AFDT | GPIO30, MCEH_B |  |  | Set field 1–0 of the Pin Mux Register B to 10. |
| BUSY | GPIO24, BVD2_B |  |  | Set field 1–0 of the Pin Mux Register B to 10. |
| ERROR | GPIO27, CD_B |  |  | Set field 1–0 of the Pin Mux Register B to 10. |
| INIT | GPIO28, REG_B |  |  | Set field 1–0 of the Pin Mux Register B to 10. |
| PE | GPIO26, RDY_B |  |  | Set field 1–0 of the Pin Mux Register B to 10. |
| SLCT | GPIO23, WP_B |  |  | Set field 1–0 of the Pin Mux Register B to 10. |
| SLCTIN | GPIO29, RST_B |  |  | Set field 1–0 of the Pin Mux Register B to 10. |
| STRB | GPIO31, MCEL_B |  |  | Set field 1–0 of the Pin Mux Register B to 10. |

| Signal You Want |  | Signals You Give Up | How to Configure the Signal You Want on the Pin |
| --- | --- | --- | --- |
| VESA Local Bus Control |  |  |  |
| VL_ADS |  | LCDD1 | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_BLAST |  | LVDD | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_BE3 |  | LCDD7 | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_BE2 |  | M | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_BE1 |  | LC | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_BE0 |  | SCK | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_BRDY |  | LVEE | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_D/C |  | LCDD5 | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_LCLK |  | FRM | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_LDEV |  | LCDD6 | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_LRDY |  | LCDD4 | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_M/IO |  | LCDD3 | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_RST |  | LCDD0 | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| VL_W/R |  | LCDD2 | Enable the VL-Bus interface by setting bit 3 of the Cache and Miscellaneous Register to 1. |
| ISA Bus |  |  |  |
| AEN | GPIO_CS10 |  | Set bit 0 of the Pin Mux Register A to 1. |
| IOCHRDY | GPIO_CS6 |  | Set bit 3 of the Pin Mux Register A to 1. |
| IOCS16 | GPIO_CS5 |  | Set bit 4 of the Pin Mux Register A to 1. |
| PDACK0 | GPIO_CS11 |  | Set bit 0 of the Pin Mux Register A to 1. |
| PDRQ0 | GPIO_CS12 |  | Set bit 0 of the Pin Mux Register A to 1. |
| TC | GPIO_CS9 |  | Set bit 0 of the Pin Mux Register A to 1. |

| Signal You Want | Signals You Give Up | How to Configure the Signal You Want on the Pin |
| --- | --- | --- |
| GPIOs |  |  |
| GPIO31 | STRB, MCEL_B | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO30 | AFDT, MCEH_B | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO29 | SLCTIN, RST_B | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO28 | INIT, REG_B | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO27 | ERROR, CD_B | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO26 | PE, RDY_B | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO25 | ACK, BVD1_B | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO24 | BUSY, BVD2_B | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO23 | SLCT, WP_B | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO22 | PPOEN | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO21 | PPDWE | Set field 1–0 of the Pin Mux Register B to 00. |
| GPIO20 | CD_A2 | Set bit 0 of the Pin Mux Register C to 0. |
| GPIO19 | LBL2 | Set bit 4 of the Pin Mux Register B to 0. |
| GPIO18 | PCMB_VPP2 | Set bit 6 of the Pin Mux Register B to 0. |
| GPIO17 | PCMB_VPP1 | Set bit 6 of the Pin Mux Register B to 0. |
| GPIO16 | PCMB_VCC | Set bit 6 of the Pin Mux Register B to 0. |
| GPIO15 | PCMA_VPP2 | Set bit 5 of the Pin Mux Register B to 0. |
| GPIO_CS14 | PCMA_VPP1 | Set bit 5 of the Pin Mux Register B to 0. |
| GPIO_CS13 | PCMA_VCC | Set bit 5 of the Pin Mux Register B to 0. |
| GPIO_CS12 | PDRQ0 | Set bit 0 of the Pin Mux Register A to 0. |
| GPIO_CS11 | PDACK0 | Set bit 0 of the Pin Mux Register A to 0. |
| GPIO_CS10 | AEN | Set bit 0 of the Pin Mux Register A to 0. |
| GPIO_CS9 | TC | Set bit 0 of the Pin Mux Register A to 0. |
| GPIO_CS8 | PIRQ0 | Set bit 1 of the Pin Mux Register A to 0. |
| GPIO_CS7 | PIRQ1 | Set bit 2 of the Pin Mux Register A to 0. |
| GPIO_CS6 | IOCHRDY | Set bit 3 of the Pin Mux Register A to 0. |
| GPIO_CS5 | IOCS16 | Set bit 4 of the Pin Mux Register A to 0. |
| GPIO_CS4 | DBUFOE | Hardwire-strap the CFG3 pin Low. |
| GPIO_CS3 | DBUFRDH | Hardwire-strap the CFG3 pin Low. |
| GPIO_CS2 | DBUFRDL | Hardwire-strap the CFG3 pin Low. |

| Signal You Want | Signals You Give Up | How to Configure the Signal You Want on the Pin |
| --- | --- | --- |
| Keyboard Interface |  |  |
| XT_CLK | KBD_COL1 | Set bit 3 of the Pin Mux Register B to 0. |
| XT_DATA | KBD_COL0 | Set bit 3 of the Pin Mux Register B to 0. |
| KBD_COL6 | PIRQ7 | Set bit 1 of the Pin Mux Register C to 0. |
| KBD_COL5 | PIRQ6 | Set bit 1 of the Pin Mux Register C to 0. |
| KBD_COL4 | PIRQ5 | Set bit 1 of the Pin Mux Register C to 0. |
| KBD_COL3 | PIRQ4 | Set bit 1 of the Pin Mux Register C to 0. |
| KBD_COL2 | PIRQ3 | Set bit 1 of the Pin Mux Register C to 0. |
| KBD_COL1 | XT_CLK | Set bit 3 of the Pin Mux Register B to 0. |
| KBD_COL0 | XT_DATA | Set bit 3 of the Pin Mux Register B to 0. |
| KBD_ROW14 | SUS_RES | Set bit 3 of the Keyboard Configuration Register A to 1. |
| KBD_ROW13 | R32BFOE | Hardwire-strap both the CFG1 and CFG0 pins Low. |
| KBD_ROW12 | MCS16 | Set bit 2 of the Pin Mux Register B to 0. |
| KBD_ROW11 | SBHE | Set bit 2 of the Pin Mux Register B to 0. |
| KBD_ROW10 | BALE | Set bit 2 of the Pin Mux Register B to 0. |
| KBD_ROW9 | PIRQ2 | Set bit 2 of the Pin Mux Register B to 0. |
| KBD_ROW8 | PDRQ1 | Set bit 2 of the Pin Mux Register B to 0. |
| KBD_ROW7 | PDACK1 | Set bit 2 of the Pin Mux Register B to 0. |
| KBD_ROW6 | MA12 | Set bit 3 of the DRAM Bank x Configuration Register to 0. |
| KBD_ROW5 | RAS3 | Set bit 3 of the DRAM Bank x Configuration Register to 0. |
| KBD_ROW4 | RAS2 | Set bit 3 of the DRAM Bank x Configuration Register to 0. |
| KBD_ROW3 | CASH3 | Set bit 3 of the DRAM Bank x Configuration Register to 0. |
| KBD_ROW2 | CASH2 | Set bit 3 of the DRAM Bank x Configuration Register to 0. |
| KBD_ROW1 | CASL3 | Set bit 3 of the DRAM Bank x Configuration Register to 0. |
| KBD_ROW0 | CASL2 | Set bit 3 of the DRAM Bank x Configuration Register to 0. |

| Signal You Want | Signals You Give Up | How to Configure the Signal You Want on the Pin |
| --- | --- | --- |
| PC Card |  |  |
| BVD1_B | GPIO25, ACK | Set field 1–0 of the Pin Mux Register B to 01. |
| BVD2_B | GPIO24, BUSY | Set field 1–0 of the Pin Mux Register B to 01. |
| CD_A2 | GPIO20 | Set bit 0 of the Pin Mux Register C to 1. |
| CD_B | GPIO27, ERROR | Set field 1–0 of the Pin Mux Register B to 01. |
| LBL2 | GPIO19 | Set bit 4 of the Pin Mux Register B to 1. |
| MCEL_A | BDNSCN_TCK | Pull the BNDSCN_EN pin Low. |
| MCEH_A | BNDSCN_TMS | Pull the BNDSCN_EN pin Low. |
| MCEL_B | GPIO31, STRB | Set field 1–0 of the Pin Mux Register B to 01. |
| MCEH_B | GPIO30, AFDT | Set field 1–0 of the Pin Mux Register B to 01. |
| PCMA_VCC | GPIO_CS13 | Set bit 5 of the Pin Mux Register B to 1. |
| PCMA_VPP1 | GPIO_CS14 | Set bit 5 of the Pin Mux Register B to 1. |
| PCMA_VPP2 | GPIO15 | Set bit 5 of the Pin Mux Register B to 1. |
| PCMB_VCC | GPIO16 | Set bit 6 of the Pin Mux Register B to 1. |
| PCMB_VPP1 | GPIO17 | Set bit 6 of the Pin Mux Register B to 1. |
| PCMB_VPP2 | GPIO18 | Set bit 6 of the Pin Mux Register B to 1. |
| REG_A | BNDSCN_TDO | Pull the BNDSCN_EN pin Low. |
| RST_A | BNDSCN_TDI | Pull the BNDSCN_EN pin Low. |
| REG_B | GPIO28, INIT | Set field 1–0 of the Pin Mux Register B to 01. |
| RST_B | GPIO29, SLCTIN | Set field 1–0 of the Pin Mux Register B to 01. |
| RDY_B | GPIO26, PE | Set field 1–0 of the Pin Mux Register B to 01. |
| WP_B | GPIO23, SLCT | Set field 1–0 of the Pin Mux Register B to 01. |
| Miscellaneous |  |  |
| BL0 | CLK_IO | Set field 7–6 of the Pin Mux Register A to 01. |
| CLK_IO | BL0 | Set field 7–6 of the Pin Mux Reg.A to 10 to enable CLK_IO as an output or to 11 to enable as a timer clock input. |
| SUS_RES | KBD_ROW14 | Set bit 3 of the Keyboard Configuration Register to 0. |

| Signal You Want | Signals You Give Up | How to Configure the Signal You Want on the Pin |
| --- | --- | --- |
| LCD Graphics Controler |  |  |
| FRM | VL_LCLK | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LC | VL_BE1 | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LCDD0 | VL_RST | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LCDD1 | VL_ADS | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LCDD2 | VL_W/R | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LCDD3 | VL_M/IO | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LCDD4 | VL_LRDY | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LCDD5 | VL_D/C | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LCDD6 | VL_LDEV | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LCDD7 | VL_BE3 | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LVEE | VL_BRDY | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| LVDD | VL_BLAST | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| M | VL_BE2 | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| SCK | VL_BE0 | Enable the graphics controller by setting bit 2 of the Internal Graphics Control Register A to 1. |
| Boundary Scan Interface |  |  |
| BDNSCN_TCK | MCEL_A | Pull the BNDSCN_EN pin High. |
| BNDSCN_TDI | RST_A | Pull the BNDSCN_EN pin High. |
| BNDSCN_TDO | REG_A | Pull the BNDSCN_EN pin High. |
| BNDSCN_TMS | MCEH_A | Pull the BNDSCN_EN pin High. |

Anhang B

miniMODUL-486 Pinout

| 151 * | 149 * | 147 * | 145 * | 143 * | 141 * | 139 * | 137 * | 135 * | 133 * | 131 * | 129 * |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| /MEMR | SA20 | SA22 | SA24 | ICDIR | /PCM _OE | /RDY_A | RST-A TDI | BVD1-A | BVD2-A | BND SCNEN | /WAIT_AB |
| SLCT | /MEMW | SA21 | SA23 | SA25 | /PCM _WE | /CD_A | /MCEL _A TCK | /MCEH_A TMS | /REG_A TDO | SCLK | WP-A |
| 152 * | 150 * | 148 * | 146 * | 144 * | 142 * | 140 * | 138 * | 136 * | 134 * | 132 * | 130 * |

| 1 | VCC5 | 2 | VCC3 |  | 1 |  | 128 | GND | 127 | GND |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | PD1 | 4 | PD0 |  | 2 |  | 126 | /PCMA_VCC | 125 | DRQ1 |
| 5 | PD3 | 6 | PD2 |  | 3 |  | 124 | /DACK1 | 123 | PCMA-VPP2 |
| 7 | PD5 | 8 | PD4 |  | 4 |  | 122 | IRQ7 | 121 | IRQ6 |
| 9 | PD7 | 10 | PD6 |  | 5 |  | 120 | IRQ5 | 119 | IRQ4 |
| 11 | /STRB | 12 | GPIO16 |  | 6 |  | 118 | /SMEMR | 117 | /SMEMW |
| 13 | GPIO17 | 14 | GPIO18 |  | 7 |  | 116 | IRQ1 | 115 | TC |
| 15 | /AFDT | 16 | /INIT |  | 8 |  | 114 | /IOCS16 | 113 | /MEMCS16 |
| 17 | /SLCTIN | 18 | PE |  | 9 |  | 112 | SA18 | 111 | SA19 |
| 19 | /ACK | 20 | /ERROR |  | 10 |  | 110 | IRQ0 | 109 | KBDROW0 |
| 21 | BUSY | 22 | GND |  | 11 |  | 108 | KBDROW1 | 107 | KBDROW2 |
| 23 | /IOW | 24 | /SBHE |  | 12 |  | 106 | KBDROW3 | 105 | KBDROW4 |
| 25 | /BAUDOUT | 26 | IOCHRDY |  | 13 |  | 104 | KBDROW5 | 103 | KBDROW6 |
| 27 | BSD14 | 28 | BSD15 |  | 14 |  | 102 | KBDROW13 | 101 | GPIO_CS11 |
| 29 | BSD12 | 30 | BSD13 |  | 15 |  | 100 | GPIO_CS12 | 99 | PCMA-VPP1 |
| 31 | BSD10 | 32 | BSD11 |  | 16 |  | 98 | KBDROW14 | 97 | RSTDRV |
| 33 | BSD8 | 34 | BSD9 |  | 17 |  | 96 | /SERENA | 95 | KBDCOL2 |
| 35 | BSD6 | 36 | BSD7 |  | 18 |  | 94 | OUT1 | 93 | SCL |
| 37 | BSD4 | 38 | BSD5 |  | 19 |  | 92 | KBDCOL0 | 91 | KBDCOL1 |
| 39 | BSD2 | 40 | BSD3 |  | 20 |  | 90 | KBDCOL2 | 89 | GND |
| 41 | BSD0 | 42 | BSD1 |  | 21 |  | 88 | SPKR | 87 | IRQ2 |
| 43 | SA14 | 44 | SA15 |  | 22 |  | 86 | /CDA2 | 85 | /PWRFAIL |
| 45 | SA12 | 46 | SA13 |  | 23 |  | 84 | PWRSENSE | 83 | /IOR |
| 47 | SA10 | 48 | SA11 |  | 24 |  | 82 | SA17 | 81 | ACIN |
| 49 | SA8 | 50 | SA9 |  | 25 |  | 80 | SA16 | 79 | /RESETIN |
| 51 | SA6 | 52 | SA7 |  | 26 |  | 78 | /BAT_LOW2 | 77 | /BAT_LOW1 |
| 53 | SA4 | 54 | SA5 |  | 27 |  | 76 | RXD232A | 75 | TXD232A |
| 55 | SA2 | 56 | SA3 |  | 28 |  | 74 | RIN232B | 73 | DCD232B |
| 57 | SA0 | 58 | SA1 |  | 29 |  | 72 | DSR232B | 71 | DTR232B |
| 59 | AEN | 60 | SIROUT |  | 30 |  | 70 | CTS232B | 69 | RTS232B |
| 61 | SIRIN | 62 | BALE |  | 31 |  | 68 | RXD232B | 67 | TXD232B |
| 63 | GND | 64 | GND |  | 32 |  | 66 | VCC3 | 65 | VCC5 |

C  Programmier- und JTAG-Ports

Es wurde mit der Beschaltung dieser Stiftleiste die Möglichkeit geschaffen bei zukünftigen Bestückungs-Varianten das System-Contoller-EPLD nachträglich, also OnBoard mit Kundenspezifischen Design-Variationen zu programmieren.

| Belegung Pin # | name |  | Pin # | name |
| --- | --- | --- | --- | --- |
| 1 | /FLT (ÉLAN™SC400) |  | 2 | TCK (ÉLAN™SC400) |
| 3 | TDI (ÉLAN™SC400) |  | 4 | /TRST (ÉLAN™SC400) |
| 5 | TMS (ÉLAN™SC400) |  | 6 | TDO (ÉLAN™SC400) |
| 7 | TDI-E (EPLD) |  | 8 | TDO-E (EPLD) |
| 9 | TMS-E (EPLD) |  | 10 | TCK-E (EPLD) |
| 11 | /SMI |  | 12 | A24 |
| 13 | RGD-BSECT |  | 14 | GND |

Belegung JTAG-Port (BND-scan)

Leitung #1

Flachbandkabel						Verbinder (Stifte)

1							     127

| JTAG | - Flachbandkabel |
| --- | --- |
| # | Signal |
| 1 | GND |
| 2 | - |
| 3 | - |
| 4 | - |
| 5 | - |
| 6 | - |
| 7 | - |
| 8 | - |
| 9 | - |
| 10 | - |
| 11 | BNDSCNEN |
| 12 | - |
| 13 | - |
| 14 | TDO |
| 15 | - |
| 16 | TMS |
| mm486 Draufsicht 17 | TDI |
| 18 | TCK |
| 19- 26 | am Verbinder abschneiden |

Zum aktivieren des JTAG-Ports wird die Leitung “BNDSCNEN“ in die Buchsen-

leiste Pin # 1 (VCC) gesteckt